stb_dcache设计说明文档

- 目的 Purpose

本文档用于指导 stb_dcache的设计的需求文档。

- 概述 Overview

  - 模块简介 Introduction

stb_dcache用在32位risc-v高性能mcu上。这个stb_dcache是位于LSU访存单元和BIU单元之间的数据缓存，LSU写请求先会进store buffer,通过解耦前端流水线和dcache下游写操作，减少store导致的流水线阻塞和反压。 tilelink IO，stb_dcache的顶层端口信号已经固定好（不可随意加减信号），参考stb_dcache.sv文件。

  - 特性列表 Feature List

- cache容量支持16/32 /64/128KB可配置（顶层dcache_def修改cachesize,要求整体stb_dcache rtl参数化设计，需要的参数化的信息已经写在dcache_def文件里面）。

- Dcache带ECC功能，ecc写在tagram和dataram的最高位。

- cache line size支持32 Byte  ,tilelink数据位宽是64bit。

- LSU写请求会发出双字，单字，半字，以及字节访问，lsu发的读请求双字读，mask 为ff，不会发非对齐访问所以不用支持非对齐

- 四路组相联。

- cacheline 的dirty位在sram，cacheline的valid_bit在dcache 内部寄存器实现，cache的sram采用外挂方式，顶层可以看到接口信号，sram需要secded ECC保护，支持错误上报，上报方式参考接口信号部分。

- 设计需要跑很高的频率, tagram和dataram的读数据进入stb_dcache后先进行ECC解码，解码后的数据必须立刻进入寄存器打一拍而不能当拍使用.

- 采用write back和write allocate策略。

- 阻塞式(blocking)设计，即一旦发生miss，则阻塞后续访问直至数据填回(refill)。

- 支持CMO处理，包括invalid/flush/clean/zero/(invalid_all/clean_all自定义的)

- 支持tilelink-H amo访问，支持LR SC（一个reservation set），其中LR (opcode== get, param==1,sc是putpart +param==1)

- 不能有sram输入到sram输出的feed-through路径.

- ECC解码器是现成的，放在ecc文件夹中，调用时参考以下图片ecc例化,ecc例化2方式

- 接口信号列表 Interface

6.1.1.3 Slv_TL_A_Chan_t

| Field Name | Type | Description |
| --- | --- | --- |
| tl_a_opcode | 3 | Opcode for the TL-A channel - 0: put full. A PutFullData message is a request made by an agent that would like to access a particular block of data in order to write it. - 1: put partial data. A PutPartialData message is a request made by an agent that would like to access a particular block of data in order to write it. Mask is used to indicate which bytes are valid - 2: arithmetic data. An ArithmeticData message is a request made by an agent that would like to access a particular block of data in order to read-modify-write it by applying an arithmetic operation. The param specific the arithmetic operation - 3: logic data. A LogicalData message is a request made by an agent that would like to access a particular block of data in order to read-modify-write it by applying a bitwise logical operation. The param specific the logical operation - 4: get. A Get message is a request made by an agent that would like to access a particular block of data in order to read it. |
| tl_a_param | 3 | Parameter for the TL-A channel - When opcode is 2 - 0: MIN, Write the signed minimum of the two operands, and return the old value. - 1: MAX, Write the signed maximum of the two operands, and return the old value. - 2: MINU, Write the unsigned minimum of the two operands, and return the old value. - 3: MAXU, Write the unsigned maximum of the two operands, and return the old value. - 4: ADD, Write the sum of the two operands, and return the old value. - When opcode is 1 - 0: XOR, Bitwise logical xor the two operands, write the result, and return the old value - 1: OR, Bitwise logical or the two operands, write the result, and return the old value. - 2: AND, Bitwise logical and the two operands, write the result, and return the old value. - 3: SWAP, Swap the two operands and return the old value. |
| tl_a_size | 3 | Size of the transfer 0: 8-bit access 1: 16-bit access 2: 32-bit access 3: 64-bit access |
| tl_a_source | 4 | Source identifier for the TL-A channel - bit0: outstanding id - bit[3:1]: master id 3’b000: IFU master 3’b001: LSU DC master 3’b010: Front port master 3’b011: DTIM master 3’b100: DEV master |
| tl_a_mask | 8 | Byte mask for the TL-A channel. Each bit in tl_a_mask indicates whether the corresponding byte in tl_a_data is valid. For example, when bit[0] of tl_a_mask is 1, it indicates that bits[7:0] of tl_a_data are valid |
| tl_a_addr | 32 | Address for the TL-A channel |
| tl_a_data | 64 | Data for the TL-A channel |
| misc | mas_tl_a_misc_t | Miscellaneous information for the TL-A channel. |

6.1.1.4 Slv_TL_D_Chan_t

| Field Name | Type | Description |
| --- | --- | --- |
| tl_d_opcode | 3 | Opcode for the TL-D channel - 0: put full. A PutFullData message is a request made by an agent that would like to access a particular block of data in order to write it. - 1: put partial data. A PutPartialData message is a request made by an agent that would like to access a particular block of data in order to write it. Mask is used to indicate which bytes are valid - 2: arithmetic data. An ArithmeticData message is a request made by an agent that would like to access a particular block of data in order to read-modify-write it by applying an arithmetic operation. The param specific the arithmetic operation - 3: logic data. A LogicalData message is a request made by an agent that would like to access a particular block of data in order to read-modify-write it by applying a bitwise logical operation. The param specific the logical operation - 4: get. A Get message is a request made by an agent that would like to access a particular block of data in order to read it. |
| tl_d_param | 3 | Parameter for the TL-D channel - When opcode is 2 - 0: MIN, Write the signed minimum of the two operands, and return the old value. - 1: MAX, Write the signed maximum of the two operands, and return the old value. - 2: MINU, Write the unsigned minimum of the two operands, and return the old value. - 3: MAXU, Write the unsigned maximum of the two operands, and return the old value. - 4: ADD, Write the sum of the two operands, and return the old value. - When opcode is 1 - 0: XOR, Bitwise logical xor the two operands, write the result, and return the old value - 1: OR, Bitwise logical or the two operands, write the result, and return the old value. - 2: AND, Bitwise logical and the two operands, write the result, and return the old value. - 3: SWAP, Swap the two operands and return the old value. |
| tl_d_size | 3 | Size of the transfer 0: 8-bit access 1: 16-bit access 2: 32-bit access 3: 64-bit access |
| tl_d_source | 4 | Source identifier for the TL-D channel - bit0: outstanding id - bit[3:1]: master id 3’b000: IFU master 3’b001: LSU DC master 3’b010: Front port master 3’b011: DTIM master 3’b100: DEV master |
| tl_d_data | 64 | Data for the TL-D channel |
| tl_d_error | 1 | Error flag for the TL-D channel. When this signal is 1, it indicates that the request has encountered an error. |

6.1.1.6 cbo_type_e[2:0]

The cbo_type_e used to indicate the cbo instruction type. The cbo instruction type indicates by the enum member name

| Enum member | Value |
| --- | --- |
| CBO_INVAL | 3'b000 |
| CBO_CLEAN | 3'b001 |
| CBO_FLUSH | 3'b010 |
| CBO_ZERO | 3'b011 |
| CBO_CLEAN_ALL | 3'b100 |
| CBO_INVAL_ALL | 3'b101 |

6.2时钟复位 Clock and Reset

| Port name | Range | Driver/Output | Description |
| --- | --- | --- | --- |
| clk_i | 1 | top_input | CPU主时钟 CPU Main Clock |
| rst_n_i | 1 | top_input | CPU复位信号, 低有效 CPU Reset Signal, Active Low |

6.3 LSU to stb_dcache

| Port name | Range | Driver/Output | Description |
| --- | --- | --- | --- |
| cbo_req_i | 1 | LSU | CBO请求有效信号 CBO Request Valid Signal |
| cbo_addr_i | 32 | LSU | CBO请求地址 CBO Request Address |
| cbo_ack_o | 1 | Output | CBO响应信号 CBO Response Signal |

  - LSU to stb_dcache

| Port name | Range | Driver /Output | Description |
| --- | --- | --- | --- |
| tl_slv_a_vld_i | 1 | lsu | TL_SLV_A通道信号，代表lsu向stb_dcache发送请求 |
| tl_slv_a_rdy_o | 1 | Output | TL_SLV_A通道信号,代表stb_dcache可以接收来自lsu的请求。 |
| tl_slv_d_vld_o | 1 | Output | TL_SLV_D通道信号，代表stb_dcache向lsu回复响应信号。 TL_SLV_D Channel Signal: Represents the response signal sent from stb_dcache to lsu. |
| tl_slv_d_rdy_i | 1 | lsu | TL_SLV_D通道信号，代表lsu可以接收响应信号。 TL_SLV_D Channel Signal: Represents that lsu is ready to receive response signals. |
| stb_dcache_drained_out | 1 | Output | Store buffer排空信号 Store Buffer Flush Signal |

  - stb_dcache to BIU(BIU)

| Port name | Range | Driver/Output | Description |
| --- | --- | --- | --- |
| dc_tl_mst_a_vld_o | 1 | Output | TL_MST_A通道信号,代表stb_dcache向下级BIU发送cacheable 的refill请求、脏数据回写， |
| dc_tl_mst_a_rdy_i | 1 | BIU | TL_MST_A通道信号,代表BIU可以接收来自stb_dcache的请求。 |
| dc_tl_mst_d_vld_i | 1 | BIU | 指示BIU向stb_dcache回复的信息是否有效 |
| dc_tl_mst_d_rdy_o | 1 | Output | 指示stb_dcache是否可以接收来自BIU的回复信息 |

  - stb_dcache to errctrl

| Port name | Range | Driver /Output | Description |
| --- | --- | --- | --- |
| check_en_i | 1 | errctrl | ECC校验结果向errctrl_top进行上报的开关。 Switch for Reporting ECC Check Results to errctrl_top |
| tag_err_vld_o | 1 | output | tagram ecc error way有效信号 Tag RAM way ECC Error Valid Signal |
| tag_err_addr_o | 19 | output | tag ram way出错的tag index。 Tag RAM way Error Tag Index |
| tag_err_type_o | 2 | output | tag ram way出错信息 0-没有错误 2’b01-ecc single error 2’b10-ecc double error 复位值为0。 Tag RAM way Error Information: 0: No error 2'b01: ECC single error 2'b10: ECC double error Reset value is 0. |
| data_err_vld_o | 1 | output | Dataram ecc error 有效信号 Data RAM ECC Error Valid Signal |
| data_err_addr_o | 18 | output | data ram出错的data index。 Data RAM Error Data Index |
| data_err_type_o | 2 | output | data ram出错信息 0-没有错误 2’b01-ecc single error 2’b10-ecc double error 复位值为0。 Data RAM Error Information: 0: No error 2'b01: ECC single error 2'b10: ECC double error Reset value is 0. |
| cbo_bus_err_o | 1 | output | 在cbo处理过程中，发生的bus error。 Bus Error Occurring During CBO Processing |
| cbo_ecc_err_o | 1 | output | 在cbo处理过程中，ecc double error。 ECC Double Error Occurring During CBO Processing |

  - stb_dcache和TAG RAM接口信号 Interface Signals for stb_dcache and TAG RAM

| Port name | Range | Driver /Output | Description |
| --- | --- | --- | --- |
| tagram_addr_o | `dcTagIdxWidth | output | Tag Ram way访问地址 复位值为0。 |
| tagram_wdata_o | `dcTagRamWidth | output | Tag Ram way写数据。 |
| tagram_rdata_i | `dcTagRamWidth | Sram | Tag Ram way读数据 |
| tagram_CEB_o | 1 | output | Tag Ram way片选信号，低有效 |
| tagram_WEB_o | 1 | output | Tag Ram way写信号，低有效 |
| tagram_wm_o | `dcTagRamWidth | output | Tag Ram way写掩码信号，高有效。 当高有效时，数据才可以被写进RAM。 各位始终保持为1。 . |

  - stb_dcache和DATA RAM接口信号stb_dcache (Data Cache) and Data RAM Interface Signals

| Port name | Range | Driver/Output | Description |
| --- | --- | --- | --- |
| dataram_0_addr_o | `dcDataIdxWidth | output | Data Ram 0访问地址 复位值为0。 Data RAM 0 Access Address Reset Value is 0. |
| dataram_0_wdata_o | `dcDataRamWidth | output | Data Ram 0写数据，复位值为0。 Data RAM 0 Write Data Reset Value is 0. |
| dataram_0_rdata_i | `dcDataRamWidth | output | Data Ram 0读数据 Data RAM 0 Read Data |
| dataram_0_CEB_o | 1 | output | Data Ram 0片选信号，低有效。 Data RAM 0 Chip Select Signal, Active Low. |
| dataram_0_WEB_o | 1 | output | Data Ram 0写信号，低有效。 Data RAM 0 Write Signal, Active Low. |
| dataram_0_wm_o | `dcDataWmWidth | output | Data Ram 0写掩码信号，高有效 高有效时，数据才可以被写进RAM。 Data RAM 0 Write Mask Signal, Active High. When active high, data can be written into the RAM. |
| dataram_1_addr_o | `dcDataIdxWidth | output | Data Ram 1访问地址 复位值为0。 Data RAM 1 Access Address Reset Value is 0. |
| dataram_1_wdata_o | `dcDataRamWidth | output | Data Ram 1写数据。 复位值为0。 Data RAM 1 Write Data Reset Value is 0. |
| dataram_1_rdata_i | `dcDataRamWidth | output | Data Ram 1读数据 Data RAM 1 Read Data |
| dataram_1_CEB_o | 1 | output | Data Ram 1片选信号，低有效 复位值为1。 Data RAM 1 Chip Select Signal, Active Low Reset Value is 1. |
| dataram_1_WEB_o | 1 | output | Data Ram 1写信号，低有效 复位值为1。 Data RAM 1 Write Signal, Active Low Reset Value is 1. |
| dataram_1_wm_o | `dcDataWmWidth | output | Data Ram 1写掩码信号，高有效 高有效时，数据才可以被写进RAM。 Data RAM 1 Write Mask Signal, Active High. When active high, data can be written into the RAM. |
| dataram_2_addr_o | `dcDataIdxWidth | output | Data Ram 2访问地址 复位值为0。 Data RAM 2 Access Address Reset Value is 0. |
| dataram_2_wdata_o | `dcDataRamWidth | output | Data Ram 2写数据。 复位值为0。 Data RAM 2 Write Data Reset Value is 0. |
| dataram_2_rdata_i | `dcDataRamWidth | output | Data Ram 2读数据 Data RAM 2 Read Data |
| dataram_2_CEB_o | 1 | output | Data Ram 2片选信号，低有效 复位值为1。 Data RAM 2 Chip Select Signal, Active Low Reset Value is 1. |
| dataram_2_WEB_o | 1 | output | Data Ram 2写信号，低有效 复位值为1。 Data RAM 2 Write Signal, Active Low Reset Value is 1. |
| dataram_2_wm_o | `dcDataWmWidth | output | Data Ram 2写掩码信号，高有效 高有效时，数据才可以被写进RAM。 Data RAM 2 Write Mask Signal, Active High. When active high, data can be written into the RAM. |
| dataram_3_addr_o | `dcDataIdxWidth | output | Data Ram 3访问地址 复位值为0。 Data RAM 3 Access Address Reset Value is 0. |
| dataram_3_wdata_o | `dcDataRamWidth | output | Data Ram 3写数据。 复位值为0。 Data RAM 3 Write Data Reset Value is 0. |
| dataram_3_rdata_i | `dcDataRamWidth | output | Data Ram 3读数据 Data RAM 3 Read Data |
| dataram_3_CEB_o | 1 | output | Data Ram 3片选信号，低有效 复位值为1。 Data RAM 3 Chip Select Signal, Active Low Reset Value is 1. |
| dataram_3_WEB_o | 1 | output | Data Ram 3写信号，低有效 复位值为1。 Data RAM 3 Write Signal, Active Low Reset Value is 1. |
| dataram_3_wm_o | `dcDataWmWidth | output | Data Ram 3写掩码信号，高有效 高有效时，数据才可以被写进RAM。 Data RAM 3 Write Mask Signal, Active High. When active high, data can be written into the RAM. |

  - stb_dcache和core的接口信号　stb_dcache (Data Cache) and Core Interface Signals

| Port name | Range | Driver/Output | Description |
| --- | --- | --- | --- |
| lr_invalid | 1 | input | 失效LR寄存器有效信号 Invalidate LR Register Valid Signal |
| dc_fsm_busy | 1 | output | Dcache无请求处理，空闲状态。 |
| Test_mode_i | 1 | input | 正常模式（test_mode_i=0):时钟门控正常工作，clk_enable=0时，clk_gated关断，节省动态功耗。 测试模式（test_mode_i=1）:en_latched=1.因为en¦¦1=1,强制时钟门控out恒等于in,即把门控时钟变为用不关断的运行自由时钟。 |

- 功能描述 Function Description （Adapted by R500A）

| test_mode_i |  |
| --- | --- |

8.3 stb_dcache amo/cmo

Description

amo 请求(包括LR,SC)，cmo请求 发送amo请求，若stb_dcache_buffer为非空,反压前级, 直到stb_dcachere buffer排空. cmo请求是lsu看到stb_dcache空了才会发送

  - stb_dcache AMO

AMO请求是字操作。当stb_dcache排空时，stb_dcache将lsu发送的AMO请求直通给stb_dcache。 AMO指令tl_slv_a_opcode : 2：ArithmeticData表示算术AMO指令请求; 3: LogicalData表示逻辑AMO 指令请求； 算术指令tl_slave_a_param: 0 MIN; 1 MAX; 2 MINU; 3 MAXU; 4 ADD。 逻辑指令tl_slave_a_param: 0 XOR; 1 OR; 2 AND; 3 SWAP。

LR SC : LR指令:tl_slv_opcode=4 , tl_slave_a_param=1; SC指令: tl_slv_opcode=1 , tl_slave_a_param=1; LR/SC操作包括两个步骤: - LR指令，寄存器寄存LR指令的地址，mask值，并返回的是该地址的数据。 - SC指令，如果自从LR操作以来对应的地址和mask位未被修改，则将一个新值写入成功，写入过程和普通store指令一致。 若SC指令失败: (1)寄存器地址必须变为无效 (2) SC的数据不必写入 (3)SC指令运行结束，不在有任何操作。 (4) SC指令返回给lsu 的slv_d_data =1; 若SC指令成功： (1) 寄存器地址必须为无效 (2) SC的数据写入到dcache (3) SC指令返回给lsu的slv_d_data =0; # LR1 SC1指令之间的指令对LRSC锁的影响:（已完成LR1指令，寄存的LR信息有效，等待SC1指令的过程中，其他指令对LRSC锁的影响） get 指令，不影响LR_SC锁，无论地址是否等于LR的地址 LR寄存地址都有效。 putfull指令，若地址与LR寄存地址相同，(FF必然改写LR对应的数值）失效寄存的LR地址，SC1失败。若地址与LR地址不相同，不影响LR寄存地址。 putpart指令，地址与LR地址相同，并且请求的mask对LR寄存的mask进行操作，则失效LR寄存地址。 例如 LR mask 0F putpart F3(失效LR寄存信号） SC mask 0F faild LR mask 0F putpart F0 (不失效LR地址） SC mask 0F success AMO指令，请求地址与LR寄存地址相同，并且AMO请求的mask对LR寄存的mask进行操作，认为失效。 例如 LR mask 0F AMO mask F0 SC 0F success LR mask 0F AMO mask 0F SC F0 faild LR2指令，寄存地址更新为新的LR2指令地址，mask也更新成最新的LR2值，寄存地址有效。SC1失败 SC2指令，会失效寄存的LR1信息，导致SC1失效。 CBO指令 ，接收任何一条CBO指令都将失效寄存的LR1信号。（无论CBO请求地址是否等于LR1的地址，都无条件失效）。 LR1寄存地址对应的cacheline被写到主存，LR1寄存的地址不受影响。(单核写到主存，refill会读上来）. LR_invalid拉高，寄存器LR地址变为无效,SC1失败

stb_dcache CBO

CBO请求包括invalid/flush/clean/zero/invalid_all/clean_all，dcache处理完，回馈应答信号cbo_ack。

invalid：把对应cacheline的valid位清零，并回馈应答信号cbo_ack。 - 若所选的cacheline无效，不访问tag，不再做清零操作，响应cbo_ack - 若invalid 请求miss, 则不需要处理，响应cbo_ack。 - 若invalid 请求hit，则将其valid位清零。响应cbo_ack。

invalid_all：把所有cacheline的valid位清零，并回馈应答信号cbo_ack。 - Invalid_all请求不访问dataram 和tagram，接收到请求后对整个有效位寄存器valid_bits_reg做清零操作，响应cbo_ack。

zero：对cacheline做写0操作，并将dirty写1，并回馈应答信号cbo_ack。

clean：把对应cacheline dirty数据写回主存，然后把dirty标志清零 - 若所选的cacheline valid无效，不再做写回操作，直接回馈应答信号cbo_ack。 - 若clean 请求miss, 则不需要处理，响应cbo_ack。 - 若clean 请求hit : - 若dirty=1，则将cache line脏数据写回主存。写回后，dirty清零。响应cbo_ack - 若dirty=0，则不需要处理，响应cbo_ack

flush：把对应cacheline dirty数据写回主存，然后把dirty标志清零,再失效有效位。相当于先完成clean, 后完成invalid操作。 - 若所选的cacheline无效，不再做写回操作，直接回馈应答信号cbo_ack。 - 若flush请求miss, 则不需要处理，响应cbo_ack。 - 若flush 请求hit : - 若dirty=1，则将cache line脏数据写回主存。写回后，dirty清零。失效有效位，再响应cbo_ack - 若dirty=0，则不需要回写脏数据，失效有效位 ，响应cbo_ack

  - stb_dcache SRAM

HSR NO: HSR_DCACHE_stb_dcache_007

Description

主存映射cache计算方法： 以dcache Port (256KByte)为例，让256kbyte的物理内存范围的数据都可以映射到stb_dcache中，stb_dcache地址DcAddrwidth 18位即满足要求。218 = 28 Kbit = 256 Kbyte。 Sram结构图如下所示(： 详细计算方式请参考dcache_def.sv